# Mean Variance Explanation

The total noise variance in a pixel is the sum of the variances from 
independent noise sources: photon noise and readout noise. This leads to the formula:

    expected_variance = (signal_level + background_level) + read_noise^2

Here is why:

1. Photon Signal (S):
   Photons arriving from a star or PSF follow a Poisson distribution. For a Poisson process,
   the variance equals the mean. Therefore, the variance due to photon signal is:

       Var_signal = S

2. Background Photons (B):
   Sky background, dark current, and thermal background also follow a Poisson process.
   Thus:

       Var_background = B

   Combined photon noise variance:

       Var_photon = S + B

For a Poisson process:

Var_photon=Mean_photon=S+B 

3. Readout Noise (RON):
   Detector electronics contribute Gaussian noise with standard deviation σ_RON, independent 
   of photon noise. Therefore:

       Var_readout = σ_RON^2

4. Total Noise Variance:
   Since photon noise and readout noise are independent, their variances add:

       Var_total = Var_photon + Var_readout
                 = (S + B) + σ_RON^2

= Mean_photon + σ_RON^2